VELALAR COLLEGE OF ENGINEERING TECHNOLOGY(AUTONOMOUS), ERODE 12

DEPARTMENT OF MATHEMATICS

COURSE CODE & TITLE: 18MAT45 & PROBABILITY AND STATISTICS

ASSIGNMENT- 5

1 The following data gives the average life in hours and range in hours of 12 samples each of 5 lamps .Draw the control charts for mean and range and comment on the state of control.

| Sample no | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Mean | 120 | 127 | 152 | 157 | 160 | 134 |
| Range | 30 | 44 | 60 | 34 | 38 | 35 |
| Sample no | 7 | 8 | 9 | 10 | 11 | 12 |
| Mean | 137 | 123 | 140 | 144 | 120 | 127 |
| Range | 45 | 62 | 39 | 50 | 35 | 41 |

2. In an integrated circuit production line,samples of 100 units are checked to electrical specifications on alternate days of month and the results declared as number of defectives are tabulated below. Draw a p-chart and a np-chart and comment.

| Sample no | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No.of Defectives | 24 | 38 | 62 | 34 | 26 | 36 | 38 | 52 | 33 | 44 |
| Sample no | 11 | 12 | 13 | 14 | 15 |  |  |  |  |  |
| No.of Defectives | 44 | 52 | 45 | 30 | 34 |  |  |  |  |  |

3. 20 Pieces of cloth out of different rolls contained respectively 1,4,3,2,4,5,6,7,2,3,2,5,7,6,4,5,2,1,3 and 8 imperfections. Ascertain whether the process is in a state of statistical control.

4. Write the role and advantages of Statistical Quality Control.